Measuring the Bike-Accessibility of Community Trees and Gardens in Vancouver Using QGIS

Introduction to QGIS

Facilitator: Matthew McKitrick

For optimal viewing in Microsoft Word, use ‘Web Layout’ by going View > Web Layout

Step 1: Gather Data

- Download the following files from Vancouver open data: https://opendata.vancouver.ca/explore

  - Vancouver city boundary (.shp File)

    - Search for ‘City Boundary’. Filename: ‘city-boundary’

    - Make sure you set the export CRS to EPSG 26910 before exporting for each file.

  - Vancouver bikeways (.shp File)

    - Search ‘bikeways’

  - Community Gardens and Fruit Trees (.shp File)

    - Search ‘gardens’

  - Move file folders to an accessible folder location, such as your home directory, project folder, or on your desktop.

Step 2: Add files to QGIS

- Set data folder as favourite in QGIS

  - Use the browser on the left to navigate and select the folder that contains your data and set it as a favorite so that it is easy to find.

- Drag and drop the files from the folder into the map area

  - For shapefiles, be sure to open the folders and drag only the .shp files into QGIS, leaving the other files in place.

  - Verify that your CRS is consistent between each layer and the project before moving on.

  - If the CRS is not consistent, or some of your shapefiles are set to EPSG 4326, you may need to redownload the files as EPSG 26910.

Step 3: Garden Layer Buffer

- Create a 250-meter buffer around each community garden/tree in the layer.

  - Vector > Geoprocessing > Buffer

    - Buffer distance = 250m

      - **** If distance is in decimal degrees, you are not in a projected CRS.

- Now we have a layer that has spatial values for a 250m ring around each tree and garden.

  - We can use this layer to assess how many trees/gardens are more than 250m away from a bikeway, and therefore are ‘inaccessible’

Step 4: Select Inaccessible Locations

- Select all buffers that don’t intersect with a bikeway

  - Vector > Research Tools > Select by Location

    - Select features from: Buffered

    - Where the Features: Intersect

    - By comparing features from: Bikeways

  - Now, right click the attribute table of ‘Buffers’ and click ‘invert selection’

    - This operation deselects all currently selected features, and selects those previously unselected.

- Select all gardens/trees that are not accessible

  - Select by location again:

    - Select Features from: Trees/gardens layer

    - Intersect

    - Compare to: Buffers

    - Select: ‘Selected features only’

- Export inaccessible trees/gardens to a separate file.

  - Right click layer > Export > Save selected features as

    - Choose a name ; save in working folder.

Step 5: Identify Inaccessibility Hotspots

- Create a heatmap of hotspots

  - First, enable the Layer Styling Panel

    - View > Panels > Layer Styling Panel

  - Select ‘heat map’ from the Symbology drop-down menu

  - Define the Radius for your layer

    - The radius is defined as the maximum distance between two points for them to be blended together on the heatmap.

      - Select Map Units and give a reasonable value, say 250m.

  - Now you have a heatmap showing densities of bike-inaccessible trees and gardens

    - But, the heatmap in is obscuring the rest of your map!

  - At the bottom of the layer styling panel, open the ‘Layer Rendering’ tab.

    - Experiment with different levels of layer opacity.

    - Experiment with Blending Modes

Step 6: Prepare and Export Your Map

- Create a new map layout by opening the layout manager

  - Project > Layout Manager

  - Select ‘Create’ and enter a name for your map layout.

- Design your layout

  - Use the   icon to add your map to the layout

    - This tool adds the current extent in your Project View to a user-drawn area in the current Map Layout View.

  - Now, there are a few things you need to add to make your map Cartographically Correct

    - Add a north arrow using the  tool

    - Add a legend using the  tool

      - Adjust the item text and remove unneeded items using the  and   buttons.

      - Uncheck ‘Auto Update’ if the options are greyed out. Auto-update ensures that the layers present in the project view will be present in the layout view.

    - Add a title using the  button.

    - Add a scale bar using the  button.

  - Export your map.

    - Select your desired export format from the top toolbar in the map layout view.

      - (Image,   SVG File, PDF File).

        - .PNG Images are usually an appropriate map export format.

    - Give your map a name and save it in an accessible location.

      - Set desired image resolution and size

- Locate the file and view your map. You’re done!